# Unlocking the Quantum3D Obsidian2 90-2440 on Windows XP: Project Report

Oct 12, 2026 ·  SlopMaster

## Summary

A locked Quantum3D Obsidian2 90-2440 now renders 3D on a Windows XP PC after one run of a script that replays the Hydro Thunder CPLD handshake through RWEverything. No game files are needed.

- The card gave no video until the handshake ran. After it, the Glide donut and borg demos and Half-Life 1 all displayed, and Half-Life 1 ran just fine.

- Eight handshake words are sent and every reply matches the formula (8/8). A single word is not enough: only the full eight-word run unlocked video.

- A fresh boot followed by one unlock.py run is the whole recipe. Earlier test commands (t1, t2, t3, one) are not required.

- The unlock is lost on any restart, warm or cold, so it must be run after every boot.

- Only the unlock tool is being open-sourced. It was tested on one machine, described below.

## Test system

Everything below was tested on one machine; results on other hardware are unknown.

| Component | Detail |
| --- | --- |
| Motherboard | Intel SE440BX-2 |
| CPU | Pentium III, 450 MHz |
| Memory | 768 MB |
| Display card | GeForce MX440 on the AGP slot |
| 3D card | Quantum3D Obsidian2 90-2440 on a PCI slot, no video pass-through |
| Board CPLD | 546-0024-03 |
| 3D card driver | PeDiniz Voodoo2 driver, v2xp (athlon) build (download) |
| Operating system | Windows XP |
| Register access tool | RWEverything 1.5.3.7, C:\Program Files\Rw-Everything\Rw.exe |
| Scripting | Python 2.7.18, 32-bit |
| Card register window | BAR0 at 0xFE000000 |
| Register state after a cold boot | 0x244 = 0x08408062, 0x248 = 0x001E7E00 |
| Speed | About 2.3 s per Rw.exe call; a full eight-word run takes a few minutes |
| Software checked | Glide donut demo, borg demo, Half-Life 1 |

Because the 90-2440 has no pass-through, the monitor is moved by hand between the MX440 and the 3D card.

## Background

The Obsidian2 90-2440 is a Voodoo2-class OEM arcade board whose CPLD (546-0024-03) keeps 3D output off until it receives a security handshake. In the Hydro Thunder cabinet, the game sends that handshake while it talks to the Diego I/O board.

The goal was to unlock the card in an ordinary Windows XP test PC, so that normal Glide software can use it, without the cabinet or the original game.

## Approach

The handshake was recovered from the game's own code, then replayed from XP and checked against the captured replies.

- The game executable was analysed. Its security callback (_security_Callback) and the two helpers that write and read the CPLD (_sst1InitSigWrite, _sst1InitSigRead) were located by their debug symbols.

- The callback was run in an emulator, which gave an exact list of the register writes it makes. The Python port was then compared with that list, write for write.

- The reply formula and the eight "video enable" words were checked against real captures.

- Register access from XP goes through RWEverything's command line. Its R32 and W32 commands were confirmed on the test PC, and several commands can be chained with ; in a single call, which makes a full run take minutes instead of hours.

- Tests were staged from read-only checks, to a write of the value a register already held, to one handshake word, to the full eight words.

## Protocol

The CPLD is reached through two registers in the card's BAR0 window, used as a bit-banged serial line.

| Register | Offset | Role |
| --- | --- | --- |
| fbiInit5 | BAR0+0x244 | bit 4 = clock, bit 27 = control enable |
| fbiInit6 | BAR0+0x248 | bits 12:11 = data line; written high or low, read back as the CPLD's answer |

A transfer is 11-bit frames, least significant bit first, each carrying a byte or'd with 0x100. A 32-bit word goes out as four frames. A read command (0x500) follows, then 32 clocked reads. The first word of a pass is preceded by a 16-byte start sequence.

Each reply is computed from the word sent and, except for the first word, the previous reply:

reply = M_1(word) \oplus base \oplus M_2(previous\ reply)

M1 and M2 are fixed 32-bit bit permutations. The base is 0x7D3E9554 for the first word of a pass and 0x511B5522 for the rest, and the M2 term is dropped for the first word. The raw 32-bit value read from the CPLD equals this result, and the game byte-swaps it before handing it to the I/O board.

The eight words sent, and the replies expected for them:

| # | Word sent | Expected reply |
| --- | --- | --- |
| 1 | 0xB7C85F52 | 0x1278EE11 |
| 2 | 0xB8AD0DEA | 0x00090000 |
| 3 | 0x050D8168 | 0x70004001 |
| 4 | 0xE23C1A36 | 0x0900FF70 |
| 5 | 0xF9DCDD9C | 0x00007000 |
| 6 | 0xA15C0538 | 0x10040500 |
| 7 | 0x842E0D10 | 0x33090760 |
| 8 | 0x274B6E18 | 0x7FF44F50 |

## Results

The eight-word handshake unlocked the card every time it matched 8/8, and nothing less did.

| Test | Result |
| --- | --- |
| Port against the original callback, in an emulator | Register writes and replies identical (4134 writes in the first comparison, 3299 in the shipped regression test) |
| Read-only access to 0x244 and 0x248 through Rw.exe | Works; both registers read back sensibly |
| Write of the value 0x244 already holds, then read back | Passes; W32 is a clean 32-bit write |
| Chaining commands with ; in one Rw.exe call | Works for 192 reads; 96 commands per call used |
| First word only (one) | Reply matched the expected 0x1278EE11, but the card stayed locked |
| Full eight words (seq) | 8/8 matched; the Glide donut demo rendered |
| Half-Life 1 after the handshake | Video displayed, ran just fine |
| Card with no handshake | No video |
| Fresh boot, seq only, no other commands | 8/8 matched, card unlocked, no Mapmem error |
| Warm restart from Windows after unlocking | Card locked again; the handshake must be rerun |

Within one boot the unlock held across applications: the donut demo and then Half-Life 1 both ran without the handshake being repeated. The script restores the two registers it touches when it ends.

## Problems met

| Problem | Cause or finding | Resolution |
| --- | --- | --- |
| Early tool printed nothing, or None | No usage text; output file location unclear | Usage text added; results go to C:\ |
| Rw.exe failed to start | Path contains spaces | Path quoted and the default install path built in |
| Possible wrong skips of register writes | Reads of 0x248 show pin state, not the last value written | Skip logic now tracks only values the tool wrote |
| Run estimated at 141 minutes | Config was missing the batch size, so every command was its own call | Settings are now built automatically on first run |
| Rw.exe calls took 9.3 s, not 2.3 s | A stray donut.exe was still running in the background and using the CPU | Kill leftover demos before testing |
| Black screen after only the first word | A single word is not enough | The full eight-word run is the unlock |
| Two system crashes | Both followed swapping the monitor cable while Windows ran; the tool was not running | Treated as a cable-swap and display-driver problem; see Known issues |

## Known issues and open questions

The Mapmem error is the one unexplained problem; the rest are behaviours to plan around.

- "Mapmem driver initialization failure". The 3dfx fxPCI library printed this on some launches of Glide programs, after earlier test commands had been run. Video still rendered once. It did not appear on a fresh boot followed by a single seq run, nor after one followed by borg. The cause is unknown; the leading idea, a leftover demo process holding the card, was ruled out as the original cause by testing.

- Re-lock on restart. The card was locked again after a warm restart from Windows, so the handshake runs after every boot. A cold power-off was not tested separately, but locking would be expected.

- Cable swaps. With no pass-through on the 90-2440, moving the monitor between cards while Windows runs crashed the test PC twice. A second monitor, an A/B VGA switch, or shutting down before moving the cable avoids it.

- Single machine. A different BAR address, chipset or driver may change behaviour. The tool accepts --bar and --rw for such cases.

- Hardware writes. The tool writes to card registers. It restores the two it touches, and a power-cycle resets the card if anything hangs.

Still to find out: the exact command or order that triggered the Mapmem error, and whether other Obsidian2 boards behave the same.

## Release

Only the part that unlocks the GPU is being published, as obsidian2-unlock.

- Included: unlock.py (setup, handshake, register restore), unlock.bat (one-click launcher), a README with setup, protocol notes and warnings, and tests that need no game files.

- Not included: the game executable, ROM images, disk dumps, reverse-engineering dumps and any private tokens or addresses.

- Credits and license: the credits are filled in, and the license is the Unlicense (public domain), chosen because the code was written with AI assistance.

## Credits and sources

This project began from existing Hydro Thunder work. Those projects target making Hydro Thunder run on any video card, while this one unlocks this specific card for general 3D use.

- HT-Diego, the Hydro Thunder / Diego I/O board project where the work was first found.

- Arcade-Museum forum thread, "Confirmed working Hydro Thunder video card security bypass", which had the Hydro Thunder script files used heavily early on to check the card's behaviour.

- PeDiniz Voodoo2 driver, v2xp (athlon), the driver used on the test PC.

- RWEverything (rweverything.com), used for register access.